| ALI HAIDER haidertalpur4@gmail.com MSc Mechanical Engineering MSc, 1. félév Miskolci Egyetem Gépészmérnöki és Informatikai Kar | MUHAMMAD HAMZA DAUD hamzadaud1100@gmail.com MSc Mechanical Engineer MSc, 3. félév Miskolci Egyetem Gépészmérnöki és Informatikai Kar |
| --- | --- |

Témavezető:

Dr. Kiss László Péter egyetemi docens, ME GÉIK

Non-linear stability studies of unsymmetrically supported arches (Aszimmetrikusan megtámasztott görbe rudak nemlineáris stabilitásvizsgálatai)

Kulcsszavak: non-linear instability; unsupported arches; 1D beams; buckling

The in-plane stability of nonhomogeneous fixed-pinned arches under an arbitrarily positioned concentrated radial load is investigated in this research. By assumption, there is no torsion and the cross-sections rotate as rigid bodies and remain
perpendicular to the centroidal axis. The one-dimensional continuous beam model is non-linear geometrically by assuming moderately large rotations and is based on the single layer Euler-Bernoulli hypothesis. The material behaviour is linearly elastic, isotropic and the material composition can vary along the thickness of the arch and is the same in every single cross-section. The related coupled static equilibrium equations are derived using a variational principle. Closed-form analytical solutions are given to these. The buckling equations are also presented and evaluated to find how some geometrical parameters affect the critical loads. Some typical equilibrium paths are also plotted. Moreover, a simple example is given to show the field distributions (radial displacement, normal displacement, bending moment, axial force, shear force) along the arch axis to analyse the effect of the load position.