| DO HUYEN ANH huyenanh.d@gmail.com Physics BSc, 5. félév Debreceni Egyetem Természettudományi és Technológiai Kar |  |
| --- | --- |

Témavezető:

Kun Ferenc egyetemi tanár, DE TTK

Failure Processes of Plastic Materials

Kulcsszavak: plastic fiber reinforced composites (PFRCs); fiber bundle model (FBM); failure avalanches; power law distribution; uniform distribution

Plastic fiber-reinforced composites (PFRCs) have garnered significant
attention for their mechanical and functional advantages, including high
strength-to-weight ratio and environmental resilience. Therefore, this
study investigate the failure processes of plastic fiber bundles under grad-
ually increasing external loads, focusing on their elastic-plastic behavior
and random yield and breaking thresholds. Using a fiber bundle model
(FBM), I analyze how macroscopic stress-strain behavior and microscopic
avalanche dynamics evolve under these conditions.

Analytical derivations and numerical simulations were employed to
explore the transition from gradual to abrupt failure as the plastic co-
efficient (α) decreases. A critical value of αc ≈ 0.0536 was identified,
marking a phase-transition-like shift where the first fiber failure triggers
catastrophic collapse. Avalanches of fiber failures were shown to follow
power-law distributions. These distributions exhibited exponents τ = 5/2
for large α and τ = 3/2 for small α. Finite-size scaling revealed critical
exponents ν = 3.98 and γ = 2.65, confirming the analogy to continuous
phase transitions.

The results provide key insights into the failure dynamics of PFRCs,
demonstrating robustness in avalanche statistics under varying disorder.
This study contributes to the understanding of material reliability and
offers predictive tools for optimizing composite material design in engi-
neering applications.