| DIÁNA LUKÁCS Environmental Engineering BSc, 9. semester University of Pannonia Faculty of Engineering |  |
| --- | --- |

Supervisors:

Dr. Péter Hajós associate professor, PE MK

Dr. Krisztián Horváth assistant professor, PE MK

Environmental and sanitary significance and chromatographic analysis of gadolinium compounds

The application of the compounds of gadolinium (Gd) - which belongs to the group of lanthanides - is becoming more and more important in different application areas. In the field of medical imaging, high-resolution MRI imaging recordings are ensured by intravenously dosed Gd-chelates, which are widely spread, because these chemicals help to determine the reliable clinical diagnosis. The microelectronic industry and the environmentally friendly refrigerators are using Gd-compounds too for solving of different problems, which is a result of the special properties of Gd. At the same time usage of these compounds requires environmental prudence.

Physical, chemical properties, applications, environmental significance and effects of the Gd-compounds were studied in the course of my work. The possibilities of ion chromatographical separation and determination of Gd-chelates were examined in regard of complex formation and protonation equilibriums. It was found that in the alkaline pH range the Gd is mainly in negative charged chelate-complex form in the solution, thus anion chromatography can be finely applicable for its analytical determination. My calculations were confirmed too by the carried out experiments. Simultaneous separation of Gd-chelates (Gd-EDTA, Gd-DTPA), the interfering effects of inorganic and organic ions and also the detection limit of the applied method were studied furthermore.

Keywords: anion chromatography, gadolinium-chelate-complex, EDTA-complex, DTPA-complex, MRI contrast agents