| BALÁZS TÓTH Chemical engineer BSc, 11. semester University of Pannonia Faculty of Engineering |  |
| --- | --- |

Supervisor:

Csilla Varga Phd assistant Professor, PE MK

Investigation of recycling methods for waste rubber

Three types of rubber recycling methods have been investigated in my paper. In the first part
maximum concentration of the recycled rubber in the basic mixture has been studied. Rubber
waste concentration in the basic mixture has been determined without decrease of the mechanical
properties. That should determine the saved amount of the raw materials. Furthermore with
minimizing the number of processing steps for introducing the waste into the basic mixture, rwa
materials with a wide particle size range could be used in order to decrease the cost of the waste
processing. In the second part applicatbility of a nonphysical-chemical devulcanization process
development has been detailed in order to decrease the emissions and simplify the process. In
the third part the mechanical properties of carbon nanotubes-reinforced rubber composites have
been investigated. Since carbon nanotubes have outstanding properties by application of them as
reinforcing material high-performance composites could be prepared. It is still too expensive for
applying in rubbers, therefore only a few studies have been available in that field. Carbon nanotubes
are difficult to work with since they agglomerate, besides there are lots of regulations because it's
harmful. Coupling agent treated carbon nanotubes have been experienced to give more balanced
performance during fatigue tensile testing than without the additive, which could be related to the
higher homogeneity of the carbon nanotubes in the rubber due to the coupling agent.