| ZSÓFIA KOMA Earth Sciences BSc BSc, 5. semester Eötvös Loránd University Faculty of Science |  |
| --- | --- |

Supervisor:

Balázs Székely associate professor, ELTE TTK

Using robust plane fitting method for separate geomorphological unit

As the data amount of available Digital Terrain Models exponentially
increases, the analysis of the surface becomes increasingly challenging.
To keep pace with this trend, effective data processing methods and data
reduction techniques are becoming more and more important.
The method used in this study is a segmentation algorithm developed at the
Vienna University of Technology. It has been used successfully for
automatic detection of buildings for 3D city modelling from LiDAR data
(Dorninger and Pfeifer, 2008). This technique seemed to be applicable for
more complex topographic features, so the robust plane fitting method has
been applied to analyse terrestrian and Martian surface.
The method is based on parameter estimation of planar segments that are
fitted to the XYZ point cloud of the DTM, so that it can tolerate presence
of numerous outliers (e.g., incised valleys).
I used different parameter-sets to model different DTM data characterized
by different resolution and different accuracy. I made models for Central Andes DTM
from SRTM data, and a Mars DTM generated from ESA Mars Express HRSC
data.
My aim was to investigate how this method works on different datasets, how
different parameter-sets influence results, and what is the geological and
geomorphologic meaning of the results.
Numerous parameter sets were tested, I analysed the residual values, and
interpreted the geological and geomorphologic meaning of the planar
facets. Finally, I compared different segmentations to separate planes
that have geological or geomorphologic meaning from the others.
The results show that the robust plane fitting method can be an effective
tool to model the surface, but the geological and geomorphologic
information we can gain from the data depend on the parameter-set and the
input dataset.