| KRISZTINA JUHÁSZ Biophysics MSc MSc, 3. semester Eötvös Loránd University Faculty of Science |  |
| --- | --- |

Supervisors:

Róbert Horváth senior research fellow, Institute for Technical Physics and Materials Science, Research Centre for Natural Sciences of the Hungarian Academy of Sciences

Sándor Kurunczi research fellow, Institute for Technical Physics and Materials Science, Research Centre for Natural Sciences of the Hungarian Academy of Sciences

Investigation of antigen-antibody interactions using optical methods

Polyethylene imine (PEI) based immobilization of antibodies is described and the concept is
proved on the label free assay of C-Reactive Protein (CRP). This novel immobilization method is
composed of a hyperbranched PEI layer which was deposited at a high pH (9.5) on the sensor surface.
The free amino groups of PEI were derivatized with neutravidin by Biotin N-hydroxysuccinimide ester
and the biotinylated anti-CRP antibody immobilized on this layer. Direct binding assay of recombinant
CRP was successfully performed in the low µg/ml concentrations using a label free optical waveguide
biosensor.