| TAMÁS RIEDL Electrical Engineering MSc, 3. semester Budapest University of Technology and Economics Faculty of Electrical Engineering and Informatics |  |
| --- | --- |

Supervisor:

Dr. Bognár György associate professor, BME VIK

Local clock distribution centres with adjustable operating frequency

In my Scientific Students’ Association work, I dealt with the research and development of a
novel local clock distribution center, which is one of the most important subcircuits of the clock
distribution networks. The recent large scale integrated circuits are synchronous implements,
therefore we need to provide the accurate clock signal for them. Due to the fact that the modern
UVLSI circuits run in high frequency (several GHz) and reach a few cm2 in size, it is a real
difficulty to get along the clock signal to the registers simultaneously. In case it fails, functional
error occurs. Reducing the dimensions, the delay becomes greater, due to the increased side
capacity (Tau=R*C). In order to solve this problem, we have to place the component circuits
(ALU, controller circuits) close to each other and build up a clock distribution network. Over
the years, many solutions have been developed to distribute the clock signal properly. In this
work, I present the structure and operation of the CDNs (Clock Distribution Networks) used in
VLSI circuits and SoC (System-on-Chip) systems in detail. The new circuit is based on an analog
frequency divider, which is based on an ILO (Injection Locked Oscillator). Section 2 describes the
structure and operation of frequency dividers, and the injection locking phenomenon. Hereinafter
I review and describe the possible implementation ways of the target circuit in detail and I present
the steps of the design of my novel implementation as well as the simulation results. I designed
the circuit with the Cadence IC 6.1 design software in 0,35 um AMS technology. I have verified
the correct functional operation with simulation results.

The new circuit I designed as a result of my research and development work is a low-power
frequency divider which is able to synchronize to a clock signal and broadly applicable.