| BENCE JANKUS Architect BSc, 5. semester Budapest University of Technology and Economics Faculty of Architecture |  |
| --- | --- |

Supervisors:

Olivér Fenyvesi professor’s assistant, BME ÉŐK

Zsuzsanna Józsa associate professor, BME ÉŐK

Lightweight aggregate concrete with recycled autoclaved aggregate

Nowadays waste handling in building industry has key importance during building and demolition works. The European Union gave out a decree to regulate waste management – according to this ordinance, from 2020 the member states should recycle 70% of the offal, meant above. My research was based on working with a new lightweight aggregate concrete that contains building waste of a frequent material as aggregate – so that it could be economical and environmental too.

It was also important purposes to reach such technical parameters those are adequate to build the structures of an ordinary building (e.g. a 4-5 storey high multi-unit family house). Therefore I am working on reduce body density, increase water tightness and heat conductional properties compared with the usual reinforced normal weight concrete.

After the literature review I started laboratory tests. First I examined the aggregate – bulk density, aggregate body density, water adsorption. With these aggregate parameters I could start the planning of concrete mixture. At the end I found a mixture recipe that had good workable consistency. I made 4 mixtures with the autoclaved aggregate and one more with quartz sand and gravel, as a reference. 24 hours after the mixing specimens were demoulded, and put under water for 6 days. After than, I will store the samples in dry conditions for 21 days. At 28 days age I can make the final tests – compressive strength, water tightness and thermal conductivity. In summary I will make an economical and a constructional model to prove the advantages and disadvantages of the recycled autoclaved aggregate.