| ZOLTÁN ÁBRAHÁM Civil Engineering BSc, 6. semester Budapest University of Technology and Economics Faculty of Civil Engineering |  |
| --- | --- |

Supervisor:

Dr. György Károlyi professor, BME TTK

Strains and stresses of photoelastic beams

Certain nematic liquid crystal elastomers, called photoelastomers, change their structure
and deform when exposed to light. The distribution of light intensity is not linear, rather,
absorbed light in the material decreases exponentially with depth. As a consequence, two
stress free axes may develop when the beam, beside light, has excentric normal load. I
supplement previous studies [1] by investigating the static behavior of excentrically loaded
photoelastics beams of symmetric cross-section under light. I systematically find the
position of the stress free axes, then show analytical and numerical results of how the
number and location of the stress free axes depend on the parameters. The equilibrium
shape of the curved photoelastics beam is also presented.

[1]: M. Warner, L. Mahadevan, Photo-induced deformations of beams, plates and films,
Phys. Rev. Lett. 92, 134302 (2004).