| KITTI FEJES Bioengineer BSc, 5. semester University of Szeged Faculty of Science and Informatics | NÓRA HOMOLYA bioengineer BSc, 7. semester University of Szeged Faculty of Science and Informatics |
| --- | --- |

Supervisor:

Cecília Hodúr prof. dr. Professor, SZTE MK

Examination of protein fractions made by enzymatic hydrolysis

Abstract

Our goal is to extract milk-protein based bioactive materials and increase
their amount. Enzymes and membrane separation were used in the experiments.
We investigated the concentration of valuable substances in milk, therefore
we filtered skim milk by ultrafiltration, we applied ultrafiltration
membranes with different cutoff filters, and we measured the nutritional
values of the samples by an infrared milk-analyzer.
The ultrafiltration fluxes were examined, the pressure-normalized and the
relative flux values, their retention values, and also the milk sugar, the
milk protein, the dry matter and the total protein content of the samples.
We also investigated the effect of different concentrations of pepsin enzyme
treatments to the parameters of skimmed milk ultrafiltration, as well as the
quantity of the extracted fractions. We have found that the ultrafiltration
of milk is an effective method for concentrating valuable substances,
especially the milk proteins. The data obtained during the ultrafiltration
after pepsin treatment shows that the enzyme treatment significantly improves
the membrane filtration flux values, that is the breakdown of proteins took
place.

The measurements were excersized at room temperature using 300 rpm stirring
speed and 0.3 MPa pressure. The initial sample volume was 200 ml. We
continued the membrane filtration in each case until the compression ratio of
2 is reached, so we obtained 100 ml of filtrate for further analysis. During
the enzyme treatment we heated the skim milk sample to 40 °C, we added
hydrochloric acid to guarantee the appropriate pH and temperature values, and
then we placed it in a 40°C water bath for 2 hours. Subsequently, the samples
were sonicated to eliminate the precipitate from the denatured proteins and
obtain a more easily filterable samples. We filtered the samples by a 10 kDa
cutoff membrane filter at 500 rpm agitation and 0.3 MPa pressure. We tested
the milk protein, milk fat, lactose, full protein and dry matter content of
the samples and also their turbidity. Two series of enzymatic treatment was
carried out using different enzim (1.5% and 2%) concentrations.