| ATTILA TÁBOROSI Enviromental Engineering BSc BSc, 11. semester University of Pannonia Faculty of Engineering |  |
| --- | --- |

Supervisor:

Róbert Kurdi PhD assistant professor, PE MK

Molecular Mechanical Modeling of Montmorillonite Nanostructures

In my work the basic goal was to exam the montmorillonite-didodecyl-dimethyl-ammonium (MMT-DDDMA) complex structure as function of CEC (Cation Exchange Capacity) and basal spacing which were determined with experimental work. I didn’t deal with different structure energies, but with the molecular modeling method I could set up an energy order between the different structures.

In the experimental work R.L. Frost and fellow workers exchanged the montmorillonite’s cations to DDDMA cations. Therefore conformation analysis needed to be done for the exchange-cation (DDDMA) to find out the most stable state.

I defined both the “empty” and intercalated MMT or rather MMT-complex grid structure as function of different cation exchange capacity. I compared the obtained structures with X-ray diffraction results.

I determined that the molecular modeling method can be used well to find out montmorillonite-complex structures which consist of big amount of atoms and the computational methods expand well the experimental results.