| PÉTER BERKI Chemistry MSc MSc, 11. semester University of Szeged Faculty of Science and Informatics |  |
| --- | --- |

Supervisors:

Klára Hernádi professor, SZTE TTIK

Zoltán Németh predoctor, SZTE TTIK

Preparation and comparative characterization of In₂O₃-MWCNT nanocomposite materials

Nowadays the applications of nanocomposites based on carbon nanotubes are widely examined. My work aimed at preparing nanocomposites of In2O3-MWCNT using a simple impregnation method. The precursor compound indium (III) chloride (InCl3) was used to cover the surface of carbon nanotubes and distilled water was used as solvent. The applied mass ratio was 4:1 (In2O3:MWCNT), and during the calcination process different temperatures were investigated: 300°C, 350°C and 400°C respectively. The product materials were characterized by X-ray diffraction (XRD), energy-dispersive X-ray spectroscopy (EDX), Raman microscopy, transmission electron microscopy (TEM) and scanning electron microscopy (SEM) techniques. The average thickness of the produced surface layer and the average sizes of the In2O3 particles were calculated with the Scherrer formula and the ImageJ-program.The results show that the temperature of heat treatment affected highly the characteristic morphology and the crystal structure of the as-prepared nanocomposite.These multiwalled carbon nanotube based composite materials are promising candidates for testing as gas sensors or catalyst.

Keywords: multiwalled carbon nanotube, indium-oxide, nanocomposites, heat treatment