| LEVENTE KERTESZ Applied Electronics Department BSc, 8. semester The Technical University of Cluj-Napoca Faculty of Electronics, Telecommunication and Information Technology |  |
| --- | --- |

Supervisor:

Peter Nemeti senior research fellow,

Location Determination System Based on Acoustic Analysis of an Impact on a Smooth Surface

Location determination systems based on acoustic measurements have a large variety of applications. The goal of the project is determining the position of an impact on a smooth surfaced board, made out of a homogeneous material, along with the presentation of the implemented mathematical model’s versatile applications.

The original idea was to build an interactive panel. To do this, a personal computer’s display image has to be projected on a vertically mounted board. After determining the impact’s location on the board a mouse click can be generated on that location’s coordinates. This way any program – originally controlled by mouse clicks – can now be controlled using the board.

Total of 4 accelerometers are placed on the rectangle-shaped board’s 4 corners. When an object hits the board a vibration signal will propagate through the board. It is important that the material of the used board is a homogeneous material, so that the vibration signal propagates at the same constant speed in each direction.

Depending on the location of the impact the vibration signal will reach each accelerometer at a different time, proportional to how close the accelerometers are to the impact point. Data from each sensor is simultaneously transmitted to a high-accuracy data acquisition device. The mathematical model used for the location determination was built based on the time-difference data from the sensors.

Besides the original idea, the mathematical model implemented in LabView programming environment can be used in any location determination system based on the time-difference of arrival of the incoming signals from the sensors placed on the reference locations.