| NORBERT LUKÁCS Engineer in Pharmaceutical Industry MSc, 4. semester Budapest University of Technology and Economics Faculty of Chemical Technology and Biotechnology |  |
| --- | --- |

Supervisor:

Dr. György Tibor Balogh Head of Laboratory, Honorary Associate Professor, Gedeon Richter Plc., Compound Profiling Laboratory

Characterization of the lipophilicity of drug candidates in early stage of drug discovery

Keywords: proton dissociation constant; partition coefficient, lipophilicity

In the last two decades due to the change in drug discovery industry practice the physico-chemical profiling of the newly synthesized drug candidates became complex characterization instead of using individual parameter. The lipophilicity is an important physico-chemical parameter which is used for a long time ago to predict the absorption of drugs and interpretation of processes in pharmacokinetics and pharmacodynamics phase. Measurement of log P can use classical shake-flask and pH-metric methods. Advantage of these methods are to provide reliable log P value, but their throughput is small and the potenciometric determination of lipophilicity requires compounds what are able to ionize. Besides these classical methods the reversed phase chromatography (RP-HPLC) technique is a good alternative. RP-HPLC method has much higher throughput, but according to our knowledge it can only describe the trends of lipophilicity and can provides approximate log P datas only.

During my work we aimed to develope an automated system using RP-HPLC based lipophilicity data (log k0,grad.), which can be suitable to determine log P values of compounds in same chemical structure with using the fewest and high-precision lipophilicity datas. To develop a model system we selected 70 CNS-specific compounds from three compound families from the compound library of Gedeon Richter Plc. Our selection was based on structural diversity (Tanimoto-index) and ionization ability. Furthermore we selected 68 „gold standard” drugs with different acid-base character too. We determined the lipophilicity of the compounds by potentiometric method and RP-HPLC technique. During the RP-HPLC method we used mobile phases with different pH due to the pKa values of measured compounds. Using the obtained results we analyzed with statictical methods in a given Tanimoto range from a given compound family how many members have to be measured by classical high-accuracy method to provide the best log P – log k0,grad. correlation. In the future we want to develope a robust model system using log k0,grad. parameters and high-accuracy log P values of further compounds. This model may be able to determine the lipophilicity of a large number of CNS-specific compounds.