| DOROTTYA IDA KELEMEN environmental engineer BSc, 7. semester University of Debrecen Faculty of Engineering |  |
| --- | --- |

Supervisor:

Dr. Mihaly Molnar assistant head of laboratory, Hertelendi Laboratory of Environmental Studies

Nuclear waste disposal facility’s atmospheric 14C emission in nearby trees

Tree ring series were collected from the vicinity of a Hungarian radioactive waste treatment

and disposal facility and from a distant background area. The 14C concentration of the

cellulose content of tree rings was measured by AMS. Data of the tree ring series from the

disposal facility was compared to the background ones for each year. The results were also

compared to the 14C data of the atmospheric 14C monitoring stations at the disposal facility and

to international backgrounds. On the basis of the results, the excess radiocarbon emission of

the disposal facility can unambiguously be detected in the tree from the repository site. This

excess is of the same order of magnitude as the radiocarbon excess of the tree rings from the

nuclear weapon test period, however, it can only be detected locally, in the site.

Knowing the results it can be stated that the background tree 3 km away from the RWTDF is

unaffected by the 14C emission of the facility, thus, it can be appropriately used as a reference

point. However, the 14C excess can be well measured in case of the tree in the facility. There were

significant differences compared to the background tree. Each jump and decrease can be

attributed to a technological process performed during the development and the reprocessing

work. The comparison of the investigated tree with the tree from the bomb peak period reflects well

that the excess emission measured in the area of the RWTDF is the same order of magnitude

as the 14C concentration of the global atmosphere in the 1960’s.

Comparing the data of the DF-tree with those of the atmospheric samplers in the facility it can

be stated that the tree rings preserve the actual atmospheric 14C concentration, however, the

14C concentration values of the tree in the facility may significantly differ from certain

atmospheric data. This phenomenon is triggered by the various local topographies and the

prevailing wind direction. The 14C content of the tree in the facility is more or less affected by

the emissions of both the disposal vaults and the technological building. To sum up, it can be

concluded that the Püspökszilágy Radioactive Waste Treatment and Disposal Facility

constructed on the basis of the Safety Series No. 15 issued in 1965 only locally affects the

environment regarding the atmospheric 14C emission.