| ÁRON SOÓS Food quality and safety engineer MSc, 1. semester University of Debrecen Faculty of Agricultural and Food Sciences and Environmental Management |  |
| --- | --- |

Supervisors:

Dávid Andrási professor's assistant, DE MÉK

Dr. Béla Kovács professor, DE MÉK

Determination of the micro-elemental content of home made wines with self-developed direct analysis by ICP-MS

The aim of the investigation is the measurement of the micro-elemental content of wines. The composition of micro-elements can be important by a few aspects. Certain elements has a positive or negative effect for the health. The identification of the geographical origin of foods, especially wines is a widespread used technique by the elemental content nowadays. Moreover the high concentration of some elements causes a negative effect in the organoleptic properties, such as Fe, Mn and Cu.

Inductively coupled plasma mass spectrometry (ICP-MS) is one of the fastest and most sensitive methods for multielemental determination of element content, having a low limit of detection.

Destructive sample preparation or standard addition is not suitable for numerous sample measurement in my opinion, because of it is time-consuming and labor-intensive. Therefore non-destructive direct analysis was used. The effect of organic components of wine was corrected or decreased.

The new method was consists of a ten-fold dilution, an aqueous and matrix-matched calibration with ethanol, moreover the measured element and internal standard pairs. This method enables a fast and accurate measurement for the concentration of Mn, Fe, Co, Ni, Cu, Zn, Sr, Cd, Ba and Pb in a relatively number of samples.

Most scientists analyze commercial wines, however the micro-elemental content of home-made wines are uncharted. Hence red wines are analyzed from Hajdúhadház and surrounding. The quality of these home-made wines are evaluated in the point of the concentration of micro-elements.