| VIKTÓRIA FORGÁCS Chemistry BSc, 8. semester University of Debrecen Faculty of Science and Technology |  |
| --- | --- |

Supervisor:

Henrietta Horváth research fellow, DE TTK

Application of Ru(II)-complexes containing water soluble phosphines for decomposition or removal of environmental pollutants

Keywords: reductive hydrodehalogenation; Ru(II)-complexes; halogenated hydrocarbons

[RuCl2(p-cimol)(pta)] was catalytic active complex in the reductive hydrodehalogenation reaction of aliphatic and aromatic halogenated hydrocarbons. The best condition was 1 mmol substrate; 0.01 mmol [RuCl2(p-cimol)(pta)] complex; 1 eq. pta; 2 mmol Na-formate; 1 mL H2O; 2 h; 80°C; and 10 bar H2. Under these conditions the conversion was 27.4% in the case of chloroform and that was 52.7% in the case of carbon tetrachloride.

Furthermore, using aromatic substrates not only reductive dehalogenation but even hydrogenation took place led to formation of cyclohexyl derivatives and cyclohexane.

In addition, [RuCl2(p-cimol)(pta)] catalyst was more effective in the dehalogenation reaction compared with Ru(II)-complexes such as [RuCl2(bmim)(p-cimol)] or [RuCl2(p-cimol)(mtppms)] which contain other water soluble phosphine in aqueous media. Since the latter complex is more soluble in organic solvents I tried the dehalogenation of carbon tetrachloride with 2 complex in methanol. Although the conversion incread the dichloromethane peak could not be seen because it was under the methanol peak.

Finally I can say that I developed 3 active Ru(II)-complexes that are efficient catalysts in hydrodehalogenation reaction.