| TEODÓRA KOVÁCS Chemical engineer MSc, 2. semester Budapest University of Technology and Economics Faculty of Chemical Technology and Biotechnology |  |
| --- | --- |

Supervisors:

Dr. Imre Miklós Szilágyi research fellow, BME VBK

Dr. István Lukács research fellow, Hungaryan Academy of Sciences Institute of Technical Physics and Materials Science

Preparation of iron tungstate (FeWO4) nanosheets by hydrothermal method

Keywords: nanostructure; photocatalysis; hydrothermal; FeWO4; WO3

Tungstate oxides and metal tungstates are widely used in catalysis, photocatalysis, gas sensing, as well as in electrochromic and photoluminescence applications. Among them, hexagonal (h-) WO3, cubic (c-) WO3 and iron tungstate (FeWO4) were in the centre of my research. It is their common feature that they are often synthetized by hydrothermal synthesis. This method uses sodium tungstate (Na2WO4) and hydrochloric acid (HCl) as starting materials. The used precursors, additives and temperature (usually 180 and 200 °C) all influence the result of the reaction. According to the literature, Na2SO4 and (NH4)2SO4 were needed as additives for the preparation of h-WO3; FeSO4 and oxalic acid for the synthesis of c-WO3; while finally (NH4)2Fe(SO4)2∙6H2O or FeCl3 for obtaining FeWO4.
The purpose of my research was to investigate the effects of different cations in the sulphate additives (Na+, NH4+, Fe2+, Fe3+) on the composition of the products. Crystal structure of the products was characterized by X-ray diffraction (XRD), their morphology by scanning and transmission electron microscopies (SEM-EDX, TEM), and the optical properties with UV-VIS spectroscopy. Finally, I examined the photocatalytic activity of the products.
All the experiments were done both at 180 and 200°C. At first, I used Na2SO4 as an additive and the product was composed of h-WO3 nanorods. Then, FeSO4 was added and in contrast with previous results the product was not c-WO3 but FeWO4, which was prepared in the form of nanosheets for the first time. Next, I studied other additives with Fe2+, Fe3+ and NH4+ cations. When Mohr’s salt [(NH4)2Fe(SO4﻿)﻿2∙6H2O)] or ammonium-iron-sulphate [(NH4)Fe(SO4)2∙12H2O] were used as additives, the products were mixtures, in which h-WO3 was detected together with other ferrous compounds (e.g. Na0.24(H3O)0.76Fe3(SO4)2(OH)6, Fe2(WO4)3. Finally, when I added FeCl3 the product was Fe2(WO4)3.
In summary, in contrast with the literature I could synthetize Fe(II)WO4 only in the presence of Fe(II)SO4. In every other case, h-WO3, Fe(III)2(WO4)3 or mixtures with different phases could be prepared. As a result of my work, for the first time we managed to prepare FeWO4 nanosheets, with a thickness of 20-30 nm and a length and width of 0.5-10 µm. Both FeWO4 nanosheets and h-WO3 nanorods had good photocatalytic activity.