| TÍMEA MOLNÁR Master in Chemical Engineering MSc, 3. semester University of Pannonia Faculty of Engineering |  |
| --- | --- |

Supervisor:

Csilla Varga Phd senior lecturer, PE MK

Effects of raw materials and processing parameters on properties of CNT/PET composites

Keywords: BET, tensile strength, bottle waste, recycling, extruder

Today plastic products are used in wide areas, they are produced in large quantities, therefore volume of plastic wastes will be increasing. In my thesis I studied effects of the raw materials quality on the mechanical and rheological properties of CNT containing waste composites.
Due to the carbon nanotubes, tensile strength and modulus of the composites have been increased independently from qualities of PET, but elongation at break has decreased.
I have also investigated the effect of different types of nanotubes on the mechanical and rheological properties in mixed waste PET based CNT containing composites. I have considered that applying the same processing parameters for the same PET chips the properties of CNT/PET composites do not change with BET surface of CNTs in the studied range.. The different types of CNTs in the given BET-surface regime have not significantly influenced the processing conditions, so the composites could be extruded and injection molded at the same parameters.
Separation of PET by their colours can not be economically managed to solve, therefore recycling has to be carried out from a mixture of differently coloured PET.I had to find such parameters for the processing of differently coloured waste PETs in order to achieve a given level of mechanical strength and to improve properties of the raw materials.