| DÁNIEL FEKETŰ Mechanical Engineering MSc, 1. semester Budapest University of Technology and Economics Faculty of Mechanical Engineering |  |
| --- | --- |

Supervisor:

Zoltán Dr. Kiss Assistant Professor, BME GPK

Examination of PLA/TCP interference screws and comparasion of removed ones

I introduced and investigated the DePuy Mitek Bio-Intrafix tibial screw and its barrel. This is a biodegradable and bioresorbable interference screw what made of poly(lactic acid) and β-tri-calcium phosphate. It is used for anterior cruciate ligament (ACL) reconstruction. As manufacturer said the screw and the barrel resorb in two or three years.

A doctor found us and reported that cysta grown at the entrance of tibia at 2% of the patient what he had to cut off. During this second surgery he found pieces of screw those did not degrade in time.

In my work I compared these explanted pieces with sterile one via thermo- and thermodynamic analysis’ (DSC, DMA), Raman spectroscopy and scanning electron microscopy (SEM). Every operation made same doctor at same place (Kastélypark Klinika, Tata). I investigated only screws and tried to figure out why did not the whole screw resorb into the human body?