| AZZAYA DORJ azaad517@gmail.com Általános orvos Osztatlan, 1. félév Debreceni Egyetem Általános Orvostudományi Kar |  |
| --- | --- |

Témavezető:

Dr. Kőszegi Zsolt egyetemi docens, DE ÁOK

A nyomásalapú “mikrovaszkuláris rezisztencia rezerv (MRR)” koncepciója és összehasonlítása a Doppler mérésből származó MRR meghatározással

Almost half of the patients with stable angina who underwent invasive coronary angiography
don’t have an obstructive epicardial lesion. In these cases, coronary microcirculation
dysfunction (CMD) could be the underlining mechanism, but the microcirculation function
is hard to assess because it’s not visible on angiograms.
Microvascular resistance reserve (MRR) has been introduced as an index for evaluating CMD
that’s independent of epicardial resistance, myocardial mass, and autoregulation. MRR is
specific for microcirculation resistance and could be a prognostic marker. Originally, MRR
is calculated by Doppler-guided or thermodilution-derived flow measurement methods.
In our study, we used a pressure-bounded approach to calculate the maximal and minimal
coronary flow reserve (CFRpb) after hydrostatic pressure error correction, and also to
determine the pressure-bounded MRR interval (MRRpb). The actual MRRp-3D was
determined by hemodynamic calculation using both the measured intracoronary pressure data
and the angiography-based 3D anatomical results by QAngio XA software.
The total number of 42 vessels with a single intermediate stenosis including a test population
of 19 subjects who had Doppler flow measurement were studied. Clinical routine invasive
coronary angiography and FFR measurement provided the required data. Pressure-bound
CFRpb and MRRpb were determined by the methods described by previous publications.
The actual CFRp-3D was calculated via the website http://coronart.unideb.hu. Statistical
analysis was done by using MedCalc Software.
The regression analysis showed the actual value of MRRp-3D could be predicted from
pressure-bounded maximal MRRpb-max by the following equation:
MRRpredicted=1.09+0.49×MRRpbmax (r2=0.70, p<0.001). A significant correlation
(r=0.54, p=0.01) was found between the gold standard Doppler MRR and the predicted MRR.
The same correlation analysis in the non-ischemic FFR subgroup was also significant
(r=0.76, p=0.01). The Bland-Altman analysis showed the mean differences between the
Doppler-measured and the predicted MRR values to be -0.02 (±1.96 SD: 0.82, -0.86).
The predicted MRR from intracoronary pressure data is a simply calculable index for
assessing CMD and showed reliability even in low-pressure gradient vessels. After
hydrostatic pressure error correction, pressure gradients detected during routine FFR
measurements can provide sufficient data for calculations to simultaneously assess macroand microvascular function.