| FICSOR ATTILA attilaficsor4@gmail.com Adattudomány MSc, 1. félév Pannon Egyetem Műszaki Informatikai Kar |  |
| --- | --- |

Témavezető:

Dr. Abonyi János egyetemi tanár, PE MK

Flow-field hálózat alapú üzemelrendezés elemzése

Kulcsszavak: Flow-Field; KNN; hálózat elemzés; gyártás elrendezés

Optimizing material flow is a crucial task in manufacturing processes, where efficiency
can have a significant impact on the production rate and quality. Identification of the
optimal path between different workstations or supply and delivery points is a
non-trivial problem.
To address this issue, I propose a flow-field-based optimization technique that can be
used to find the optimal path in manufacturing processes. The fundamental concept
behind the flow-field-based path finding technique is to find optimal paths to every
potential points and aggregate the direction vectors in a flow field. The flow field-based
approach is often used in situations where there are huge amount of agents required to
be moved. Agents follow a vector based on the calculated cost-field, and as a result a
quasi-optimal path is discovered.
In this work I use a novel approach, where I use image processing, in order to
dynamically load layouts as images, and construct a mesh grid based on the content of
the input image. I remove the points that were too close to the not-allowed regions
using a nearest neighbor approach. I construct a k-nearest neighbor graph, in order to
traverse it more efficiently, and we calculate the geodesic distances as shortest paths
between all pairs of vertices. At the final step of the method, the resulting paths are
used to generate a network-based flow field that is analyzed by the tools of the network
and data science.
To demonstrate the effectiveness of the proposed technique, I applied it to a real-world
example connected to a manufacturing environment of the Operator 4.0 laboratory at
the University of Pannonia. The results demonstrate that the proposed technique can
efficiently explore critical paths and regions of the shop floor. The technique can be used
in lean projects to optimize material flow, workstation layouts, and supply and delivery
directions.